IP Addressing and Subnetting

1. IP Address Classes (A, B, C, D, E)

| Class | IP Address Range | Real-World Example |
| --- | --- | --- |
| Class A | 1.0.0.0 – 126.255.255.255 | Large networks, such as a large company's private network using 10.0.0.0. |
| Class B | 128.0.0.0 – 191.255.255.255 | College networks using private addresses such as 172.16.0.0. |
| Class C | 192.0.0.0 – 223.255.255.255 | Home or small-office LAN using 192.168.1.10. |
| Class D | 224.0.0.0 – 239.255.255.255 | Multicast streaming or video distribution. |
| Class E | 240.0.0.0 – 255.255.255.255 | Reserved for experimental or future use. |

Note: These are traditional IPv4 classes. The ranges 127.x.x.x are reserved for loopback, and Class D and E are not used for ordinary host addressing.

2. Calculate Network Address, Broadcast Address, and Valid Hosts

Given:

- IP address: 192.168.10.45

- Subnet mask: 255.255.255.0

- CIDR: /24

Step 1: Find the network address

The first three octets identify the network because the subnet mask is 255.255.255.0.

192.168.10.45 → Network address: 192.168.10.0

Step 2: Find the broadcast address

The last octet is all 1s for the broadcast address in a /24 subnet.

192.168.10.255 → Broadcast address: 192.168.10.255

Step 3: Find the valid host range

- Total host bits = 8

- Total addresses = \(2^8 = 256\)

- Usable hosts = \(256 - 2 = 254\)

Final answer:

| Item | Answer |
| --- | --- |
| Network address | 192.168.10.0 |
| Broadcast address | 192.168.10.255 |
| First valid host | 192.168.10.1 |
| Last valid host | 192.168.10.254 |
| Total usable hosts | 254 |

3. Subnetting Plan for a Food Delivery Startup:

Given network: 10.0.0.0/24

We need 4 separate subnets.

Step 1: Calculate the new prefix

- Required subnets = 4

- Borrow 2 bits because \(2^2 = 4\).

- New prefix = /24 + 2 = /26

- Subnet mask = 255.255.255.192

- Hosts per subnet = \(2^6 - 2 = 62\)

| Department | Network Address | Subnet Mask | Usable Host Range | Broadcast |
| --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1–10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65–10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129–10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.192/26 | 255.255.255.192 | 10.0.0.193–10.0.0.254 | 10.0.0.255 |

Result: The /24 network is divided into 4 equal subnets, each supporting 62 usable host addresses.

4. College Wi-Fi Network: At Least 20 Subnets:

Given network: 172.16.0.0/16

Step 1: Calculate the bits to borrow

We need at least 20 subnets.

- \(2^4 = 16\), which is not enough.

- \(2^5 = 32\), which is enough.

Therefore, borrow 5 bits.

Step 2: Find the new prefix

New prefix = /16 + 5 = /21

Step 3: Calculate the subnet mask

/21 means 21 network bits.

Subnet mask = 255.255.248.0

Step 4: Calculate usable hosts per subnet

- Host bits = \(32 - 21 = 11\)

- Total addresses = \(2^{11} = 2048\)

- Usable hosts = \(2048 - 2 = 2046\)

Final answer:

| Item | Result |
| --- | --- |
| Original network | 172.16.0.0/16 |
| Required subnets | At least 20 |
| Bits borrowed | 5 |
| Number of subnets | 32 |
| New CIDR | /21 |
| New subnet mask | 255.255.248.0 |
| Usable hosts per subnet | 2046 |

5. Why Is 192.168.1.300 Invalid?

An IPv4 address has four octets separated by dots. Each octet must contain a number from 0 to 255.

In 192.168.1.300, the last octet is 300, which exceeds the maximum of 255. Therefore, it is an invalid IPv4 address.

For the 192.168.1.0/24 subnet, examples of valid host addresses include:

- 192.168.1.1

- 192.168.1.50

- 192.168.1.254

The network address is 192.168.1.0, and the broadcast address is 192.168.1.255, so these two addresses cannot normally be assigned to individual hosts in this subnet.