VLAN Configuration

1. Network Diagram with VLANs

Cisco Switch

VLAN 10 and VLAN 20

VLAN 10 — Streaming

Smart TV

Switch port Fa0/1

VLAN 20 — Personal

Laptop

Switch port Fa0/4

Friend's phone*

Through a Wi-Fi access point

The phone connects wirelessly through an access point connected to the switch. The access point must be configured for VLAN 20.

Explanation: VLANs logically separate devices on the same physical switch. Devices in different VLANs cannot communicate directly without a router or Layer 3 switch routing between them.

2. Create VLAN 10 and VLAN 20 in Cisco Packet Tracer

Step 1: Add devices

Add one Cisco 2960 switch and five PCs.

- PC1, PC2, PC3 → VLAN 10

- PC4, PC5 → VLAN 20

Connect each PC to the switch using Copper Straight-Through cables.

Step 2: Configure the switch

Open the switch → CLI and enter:

enable

configure terminal

vlan 10

name Streaming

exit

vlan 20

name Personal

exit

interface range fastEthernet 0/1-3

switchport mode access

switchport access vlan 10

exit

interface range fastEthernet 0/4-5

switchport mode access

switchport access vlan 20

exit

end

Step 3: Verify VLANs

show vlan brief

Expected result:

| VLAN | Name | Ports |
| --- | --- | --- |
| 10 | Streaming | Fa0/1, Fa0/2, Fa0/3 |
| 20 | Personal | Fa0/4, Fa0/5 |

Step 4: Test connectivity

Assign IP addresses in the same subnet to PCs within each VLAN, for example:

- PC1: 192.168.10.1/24

- PC2: 192.168.10.2/24

- PC4: 192.168.20.1/24

From PC1, open Command Prompt and run:

ping 192.168.10.2

This should succeed if the devices and links are configured correctly.

Now run:

ping 192.168.20.1

This should fail because VLAN 10 and VLAN 20 are separate and no inter-VLAN routing has been configured.

3. Move a PC from VLAN 10 to VLAN 20

Suppose PC2 is connected to port Fa0/2.

Before: PC2 belongs to VLAN 10

enable

configure terminal

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

end

show vlan brief

After: Move PC2 to VLAN 20

enable

configure terminal

interface fastEthernet 0/2

switchport access vlan 20

end

show vlan brief

Test the change:

- Give PC2 an IP address in VLAN 20, such as 192.168.20.2/24.

- Ping PC4 at 192.168.20.1. The ping should succeed if both PCs are correctly configured.

- Ping PC1 at 192.168.10.1. The ping should fail without inter-VLAN routing.

The PC's switch port has moved from VLAN 10 to VLAN 20. Its IP address must also match the new subnet.

- Automate Assigning 10 Ports to VLAN 30

In Cisco Packet Tracer, use the interface range command to configure ten ports together.

enable

configure terminal

vlan 30

name Office

exit

interface range fastEthernet 0/1-10

switchport mode access

switchport access vlan 30

exit

end

show vlan brief

Explanation:

- vlan 30 creates VLAN 30.

- name Office gives it a name.

- interface range fastEthernet 0/1-10 selects ten switch ports.

- switchport mode access configures them as access ports.

- switchport access vlan 30 assigns all ten ports to VLAN 30.

- show vlan brief verifies the configuration.

This method is faster than configuring all ten ports individually.