Network Routing

1. Network Diagram with IP Addresses

Wi-Fi Router

LAN IP: 192.168.1.1

WAN IP: 203.0.113.10

My Phone

192.168.1.10

Friend's Laptop

192.168.1.20

Smart TV

192.168.1.30

Explanation: The router connects the devices to the local network and provides Internet access. The three devices use different private IP addresses in the same 192.168.1.0/24 subnet.

The WAN address shown is a documentation-only example, not a real public address.

2. How a Packet Travels Between Different Subnets

Suppose my laptop sends data to a friend's computer on another subnet.

- My laptop: 192.168.1.10/24

- Default gateway: 192.168.1.1

- Friend's computer: 192.168.2.20/24

Step-by-step process

- My laptop checks whether 192.168.2.20 belongs to its own subnet. It does not.

- The laptop sends the packet to its default gateway, 192.168.1.1.

- The router checks its routing table and finds a route to 192.168.2.0/24.

- The router forwards the packet through the interface connected to that subnet.

- The friend's computer receives the packet if the network and firewall permit it.

Example routing table:

| Destination Network | Next Hop | Interface |
| --- | --- | --- |
| 192.168.1.0/24 | Directly connected | LAN 1 |
| 192.168.2.0/24 | Directly connected | LAN 2 |
| 0.0.0.0/0 | 203.0.113.1 | WAN |

The router selects the most specific matching route. In this example, the destination network is directly connected, so no additional router is needed.

3. Routing Table for a Zomato-Style Delivery Network

Imagine a delivery company has three branches in different cities, connected through routers.

| Branch | Network Address | Subnet Mask | Next Hop | Interface |
| --- | --- | --- | --- | --- |
| Ahmedabad | 192.168.10.0 | 255.255.255.0 | Directly connected | G0/0 |
| Surat | 192.168.20.0 | 255.255.255.0 | 10.0.0.2 | S0/0/0 |
| Mumbai | 192.168.30.0 | 255.255.255.0 | 10.0.0.6 | S0/0/1 |

Explanation: Each branch has a separate subnet. Routers use the routing table to forward order information and other network traffic to the correct branch.

4. Static Routing vs Dynamic Routing

| Type | 3 Advantages (Pros) | 3 Disadvantages (Cons) |
| --- | --- | --- |
| Static Routing | 1. Simple for small networks. 2. No routing update traffic. 3. Gives the administrator control over routes. | 1. Routes must be configured manually. 2. Difficult to maintain in large networks. 3. Does not automatically adapt to link failures. |
| Dynamic Routing (RIP/OSPF) | 1. Learns routes automatically. 2. Can adapt when network paths change. 3. Suitable for larger networks with multiple branches. | 1. Uses CPU and memory. 2. Routing updates consume bandwidth. 3. Requires more knowledge to configure and troubleshoot. |

Real-world example: A food delivery company with branches in several cities could use OSPF to learn routes automatically. If a network link fails, OSPF can calculate an alternative route when one is available.

5. OSPF Configuration Between Two Routers

Example topology:

Router 1

LAN: 192.168.1.0/24

Link IP: 10.0.0.1/30

OSPF Area 0 — 10.0.0.0/30

Router 2

Link IP: 10.0.0.2/30

LAN: 192.168.2.0/24

Router 1 configuration

enable

configure terminal

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface Serial0/0/0

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

end

Router 2 configuration

enable

configure terminal

interface GigabitEthernet0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface Serial0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

router ospf 1

network 192.168.2.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

end

These interface names are examples; use the interfaces available on your Packet Tracer routers.

Explanation of the commands

| Command | Purpose |
| --- | --- |
| enable | Enters privileged EXEC mode. |
| configure terminal | Enters global configuration mode. |
| interface GigabitEthernet0/0 | Selects the LAN interface. |
| ip address ... | Assigns an IPv4 address and subnet mask. |
| no shutdown | Enables the interface. |
| router ospf 1 | Starts OSPF process 1. |
| network ... area 0 | Enables OSPF on matching interfaces in Area 0. The wildcard mask identifies matching addresses. |
| end | Returns to privileged EXEC mode. |

Verify the configuration

Run these commands on each router:

show ip ospf neighbor

show ip route ospf

show ip protocols

If the interfaces and OSPF configuration are correct, the routers should establish an OSPF neighbor relationship and learn each other's LAN routes.