1. Capture Flipkart traffic in Wireshark

Steps:

- Open Wireshark and select your active Wi-Fi or Ethernet interface.

- Click Start Capture.

- Open your browser and refresh Flipkart.

- Return to Wireshark and click the red Stop button.

- Select packets and examine the packet details in the middle panel.

Example packets you might identify:

| TCP/IP layer | Example protocol | What to look for |
| --- | --- | --- |
| Application | DNS or TLS | DNS query details or TLS handshake/data |
| Transport | TCP | Source port, destination port, sequence number |
| Internet | IPv4 or IPv6 | Source IP and destination IP addresses |
| Network Access | Ethernet II or 802.11 | Source and destination MAC addresses, if available |

Important: One captured packet may contain headers for several layers at once. Expand the relevant protocol sections in Wireshark to inspect them. If your browser uses QUIC, you may see QUIC over UDP instead of TCP for some web traffic.

2. Comparison of OSI and TCP/IP models

| OSI Model | TCP/IP Model | Real-world example |
| --- | --- | --- |
| 7. Application | Application | Opening Flipkart in Chrome |
| 6. Presentation | Application | Encrypting HTTPS data |
| 5. Session | Application | Maintaining an app communication session |
| 4. Transport | Transport | TCP/UDP carries application data |
| 3. Network | Internet | IP addresses route data to a server |
| 2. Data Link | Network Access | Wi-Fi or Ethernet sends frames locally |
| 1. Physical | Network Access | Wi-Fi radio signals or network cables |

Conclusion: The OSI model has 7 layers, while the TCP/IP model has 4 layers. The TCP/IP model combines the OSI Application, Presentation, and Session layers into one Application layer. It also combines the Data Link and Physical layers into Network Access.

3. TCP segment header fields

These are common fields found in a TCP header. Label them on the screenshot provided by your trainer.

| TCP header field | Function |
| --- | --- |
| Source Port | Identifies the sending application's communication port. |
| Destination Port | Identifies the receiving application's communication port. |
| Sequence Number | Identifies the position of data in the TCP byte stream. |
| Acknowledgment Number | Indicates the next byte expected from the other device when acknowledgment is used. |
| Header Length (Data Offset) | Specifies the length of the TCP header. |
| Flags | Control communication, such as SYN to start a connection and ACK to acknowledge data. |
| Window Size | Advertises how much data the receiver can accept. |
| Checksum | Helps detect errors in the TCP header and data. |
| Urgent Pointer | Indicates urgent data when the URG flag is set. |
| Options | Contains optional settings, such as maximum segment size. |

Note: All these fields belong to the TCP Transport layer. The exact fields visible depend on the header diagram.

4. Diagram: Message sent from your device to Instagram's server

1. Application Layer

I type a message or open Instagram.

2. Transport Layer

TCP or UDP carries the app's data. TCP can divide the data into segments.

3. Internet Layer

IP adds source and destination IP addresses to route the data.

4. Network Access Layer

Data is framed and sent over Wi-Fi or Ethernet.

Instagram Server

The receiving system processes the data through the layers in reverse order.

Explanation: When I use Instagram, the application creates data, the transport protocol carries it, IP helps route it across networks, and the network access layer sends it over my local connection. The receiving system processes the data to deliver the requested content.

5. Identify one packet from Wireshark

Example answer — replace these details with a packet from your capture.

- Protocol: TCP

- TCP/IP Layer: Transport Layer

- How I identified it: I selected a packet in Wireshark and expanded the TCP section. It showed the source port, destination port, and sequence number.

- Real-world action: This packet may carry part of the data exchanged between my browser and Flipkart while the homepage loads.

Conclusion: Wireshark helps me inspect network packets and understand how applications communicate over the internet.