1. Create two VLANs: Sales and Support

Open Cisco Packet Tracer and add one 2960 switch.

Click the switch → CLI, then enter:

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

Result:

- VLAN 10 = Sales

- VLAN 20 = Support

2. Assign ports and verify VLANs

Assign three ports to Sales and two ports to Support.

interface range fastEthernet 0/1-3

switchport mode access

switchport access vlan 10

exit

interface range fastEthernet 0/4-5

switchport mode access

switchport access vlan 20

exit

Verify the configuration:

end

show vlan brief

Expected result:

| VLAN | Name | Assigned ports |
| --- | --- | --- |
| 10 | Sales | Fa0/1, Fa0/2, Fa0/3 |
| 20 | Support | Fa0/4, Fa0/5 |
| 1 | default | Remaining unassigned access ports |

Take a screenshot of the CLI output for your assignment.

3. Connect two routers and configure a static route

Use this example topology:

LAN 1

192.168.1.0/24

Router 1

192.168.1.1

Router link: 10.0.0.1 ↔ 10.0.0.2

Router 2

192.168.2.1

LAN 2

192.168.2.0/24

Use Ethernet interfaces for the router-to-router link in this example. Interface names may differ by router model.

Router 1 configuration

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end

Router 2 configuration

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

end

Verify the routes:

show ip route

The LAN devices also need correct IP addresses and default gateways. For Router 1's LAN, use gateway 192.168.1.1; for Router 2's LAN, use gateway 192.168.2.1.

4. Test communication between Sales and Support VLANs

Test: Connect one PC to VLAN 10 and another PC to VLAN 20. Assign IP addresses such as:

| Device | VLAN | IP address |
| --- | --- | --- |
| PC-Sales | 10 | 192.168.10.2/24 |
| PC-Support | 20 | 192.168.20.2/24 |

From PC-Sales, run:

ping 192.168.20.2

Expected result: The ping fails when only Layer 2 VLANs are configured, because each VLAN is a separate broadcast domain and no Layer 3 routing is configured between them.

How to allow communication:

- Router-on-a-stick: Connect a router to the switch using a trunk port, configure router subinterfaces for VLANs 10 and 20, and assign each subinterface an IP gateway.

- Layer 3 switch: Create VLAN interfaces (SVIs), assign gateway IP addresses, and enable ip routing.

After configuring routing and correct PC gateways, the two PCs should be able to communicate, unless an ACL or other configuration blocks the traffic.