1. Divide 192.168.10.0/24 into 4 equal subnets

To create 4 equal subnets, borrow 2 bits because \(2^2=4\).

- Original CIDR: /24

- New CIDR: /26

- Subnet mask: 255.255.255.192

- Total subnets: 4

- Addresses per subnet: 64

- Usable hosts per subnet: 62

| Subnet | Network address | Usable IP range | Broadcast address |
| --- | --- | --- | --- |
| 1 | 192.168.10.0/26 | 192.168.10.1–62 | 192.168.10.63 |
| 2 | 192.168.10.64/26 | 192.168.10.65–126 | 192.168.10.127 |
| 3 | 192.168.10.128/26 | 192.168.10.129–190 | 192.168.10.191 |
| 4 | 192.168.10.192/26 | 192.168.10.193–254 | 192.168.10.255 |

All four subnets use the same subnet mask: 255.255.255.192.

2. Assign IP addresses to 5 devices

We will use Subnet 1: 192.168.10.0/26.

| Device | Assigned IP address |
| --- | --- |
| Router (gateway) | 192.168.10.1 |
| Laptop 1 | 192.168.10.2 |
| Laptop 2 | 192.168.10.3 |
| Printer | 192.168.10.4 |
| Mobile phone | 192.168.10.5 |

Subnet mask for all devices: 255.255.255.192

Default gateway for all devices: 192.168.10.1

These are example static addresses. Make sure they do not conflict with addresses already assigned by your router's DHCP service.

3. Subnet Plan Diagram

Main Network: 192.168.10.0/24

Subnet Mask: 255.255.255.0

Subnet 1 — /26

Network: .0

Hosts: .1–.62

Broadcast: .63

Router: .1

Laptop 1: .2

Laptop 2: .3

Printer: .4

Mobile: .5

Subnet 2 — /26

Network: .64

Hosts: .65–.126

Broadcast: .127

Other devices

Subnet 3 — /26

Network: .128

Hosts: .129–.190

Broadcast: .191

Other devices

Subnet 4 — /26

Network: .192

Hosts: .193–.254

Broadcast: .255

Other devices

4. Step-by-step subnetting explanation

- Given network: 192.168.10.0/24

- Find the required subnets: We need 4 subnets.

- Borrow bits: \(2^2=4\), so we borrow 2 bits.

- Calculate new CIDR: /24 + 2 = /26.

- Find subnet mask: /26 = 255.255.255.192.

- Calculate block size: \(256-192=64\).

- Find network addresses: Start at 0 and add 64 each time: 0, 64, 128, 192.

- Find broadcast addresses: The address immediately before the next subnet is the broadcast address: 63, 127, 191, 255.

- Find usable hosts: The first usable address is network + 1, and the last is broadcast − 1.

- Assign device IPs: In Subnet 1, assign .1 to the router, .2 and .3 to the laptops, .4 to the printer, and .5 to the mobile.

Conclusion: The /24 network is divided into four /26 subnets, each with 64 total addresses and 62 usable host addresses.