1. Ping www.youtube.com

Command (Windows CMD):

ping www.youtube.com

Example output:

Reply from 142.250.xxx.xxx: time=25ms

Reply from 142.250.xxx.xxx: time=30ms

Reply from 142.250.xxx.xxx: time=27ms

Reply from 142.250.xxx.xxx: time=26ms

Average response time = 27 ms

Answer: The ping command tests connectivity to YouTube and measures round-trip response time. In this example, the average is 27 ms. Your actual result may be different.

GitHub+1

2. Trace the route to Flipkart

Command:

tracert www.flipkart.com

This displays the intermediate network hops between your computer and the destination.

Example format:

Tracing route to www.flipkart.com

1    2 ms    1 ms    2 ms    192.168.1.1

2   10 ms   12 ms   11 ms    ISP gateway

3   18 ms   20 ms   19 ms    Intermediate router

4    *       *       *       Request timed out

Answer: Each numbered line represents a hop. A * or timeout means that hop did not respond within the time limit; it does not necessarily mean the route has failed. List the actual hops from your own output.

3. Find Zomato's IP address using nslookup

Command:

nslookup www.zomato.com

Example output format:

Server:  your-DNS-server

Address: your-DNS-IP

Name:    www.zomato.com

Address: example-IP-address

Answer: The nslookup command queries DNS to find the IP address associated with www.zomato.com. Record the actual address shown on your laptop.

4. Ping another device on your local network

First, find the other device's IP address. For example, if your friend's laptop has IP 192.168.1.5, run:

ping 192.168.1.5

If the device responds, you may see:

Reply from 192.168.1.5: time=2ms

If it is offline or not responding, you may see:

Request timed out.

Answer: When the device is offline, the ping requests may time out and no reply is received. However, a timeout can also happen because of firewall settings or disabled ping responses.

5. Short report (5–7 lines)

- Ping checks whether a destination responds and measures round-trip time.

- I used ping www.youtube.com to test connectivity and record the average response time.

- Traceroute helps identify the route and intermediate hops to a website.

- I used tracert www.flipkart.com to examine the network path.

- Nslookup helps diagnose DNS problems by checking the IP address associated with a domain.

- I used nslookup www.zomato.com to look up its IP address.

- These tools help investigate connectivity, slow responses, routing, and DNS issues.

Important: Before submitting, replace the example outputs with your actual results and screenshots.