1. RJ-45 connector pin layout: T568A and T568B

An RJ-45 (8P8C) Ethernet connector has 8 pins. Hold the plug with the gold contacts facing you and the clip facing away/down; pin 1 is on the left.

Ethernet wiring color codes

Pin order: 1 → 8, left to right

T568A

1

White/Green

2

Green

3

White/Orange

4

Blue

5

White/Blue

6

Orange

7

White/Brown

8

Brown

T568B

1

White/Orange

2

Orange

3

White/Green

4

Blue

5

White/Blue

6

Green

7

White/Brown

8

Brown

White-striped wires have a white base with a colored stripe. Solid-color wires are fully colored.

Remember: T568A and T568B differ in the positions of the green and orange pairs. A straight-through cable uses the same standard at both ends, such as T568B on both ends.

2. Comparison of Ethernet standards

| Specification | 10BASE-T | 100BASE-TX | 1000BASE-T |
| --- | --- | --- | --- |
| Speed | 10 Mbps | 100 Mbps | 1 Gbps (1000 Mbps) |
| Cable type | Cat3 or better UTP | Cat5 or better UTP | Cat5e or better UTP |
| Maximum length | 100 metres | 100 metres | 100 metres |
| Connector | RJ-45 (8P8C) | RJ-45 (8P8C) | RJ-45 (8P8C) |
| Common use | Older Ethernet networks | Older LANs and some devices | Modern home and college LANs |

UTP means Unshielded Twisted Pair. The 100-metre limit generally applies to a standard Ethernet copper link.

3. Connecting two laptops directly

Traditional answer: Use an Ethernet crossover cable.

A crossover cable swaps the transmit and receive pairs at one end, allowing the transmitting pair of one laptop to connect to the receiving pair of the other laptop.

- Straight-through cable: Same wiring standard at both ends, such as T568B–T568B.

- Crossover cable: T568A at one end and T568B at the other end, crossing the transmit and receive pairs.

Note: Many modern laptops have Auto MDI-X, which automatically adjusts transmit and receive pairs. With these laptops, a straight-through cable may also work.

4. Three devices that use Ethernet

| Device | Likely Ethernet standard | Reason |
| --- | --- | --- |
| Home Wi-Fi router | 1000BASE-T | Many modern routers have Gigabit LAN ports. |
| College desktop computer | 1000BASE-T | Common on modern desktop network adapters. |
| Network printer | 100BASE-TX or 1000BASE-T | Depends on the printer's age and model. |

These are typical examples; the exact standard depends on the device's specifications.