1. Install Wireshark and capture packets

Steps:

- Download Wireshark from the official website: wireshark.org.

- Install it on your laptop. Follow the installer instructions.

- Open Wireshark and select your active Wi-Fi or Ethernet network interface.

- Click Start Capturing Packets.

- Open a browser and visit https://www.instagram.com.

- Return to Wireshark and click the red Stop button.

- Check the Protocol column in the packet list.

Example protocols you may see:

| Protocol | Purpose |
| --- | --- |
| DNS | Finds the IP address for a website name |
| TCP | Provides reliable data delivery |
| TLS | Encrypts HTTPS communication |
| QUIC | Supports modern web connections |
| UDP | Carries datagrams between applications |
| ARP | Helps find a device's MAC address on a local IPv4 network. |

2. Identify 3 packets and their OSI layers

Example table (use your actual Wireshark packets if they differ).

| Packet | Protocol | OSI Layer | Explanation |
| --- | --- | --- | --- |
| 1 | DNS | Layer 7 – Application | Finds the IP address of a website. |
| 2 | TCP | Layer 4 – Transport | Provides reliable data delivery. |
| 3 | TLS | Layer 6 – Presentation (closest match) | Encrypts data sent through HTTPS. |

Note: Wireshark protocols do not always map perfectly to one OSI layer. TLS is commonly associated with Presentation Layer concepts, while TCP belongs to Transport Layer.

3. Colorful OSI Model diagram

You can draw this diagram in MS Paint, Canva, or on paper. Use a different color for each layer.

7

APPLICATION

WhatsApp, Instagram, Chrome

6

PRESENTATION

Image formats, encryption, UTF-8

5

SESSION

Managing a communication session

4

TRANSPORT

TCP, UDP

3

NETWORK

IP addresses, routers

2

DATA LINK

Ethernet, MAC addresses, switches

1

PHYSICAL

Wi-Fi radio signals, cables

Diagram title: The 7 Layers of the OSI Model

4. How a WhatsApp message travels through the OSI Model

When I send a message on WhatsApp, it travels through several OSI layers:

- Application Layer (Layer 7): I type a message and press Send in WhatsApp.

- Presentation Layer (Layer 6): The data is represented in a suitable format and encrypted to help protect its contents.

- Transport Layer (Layer 4): Transport protocols carry the data between devices and services.

- Network Layer (Layer 3): IP addresses help route the data across networks toward its destination.

- Data Link Layer (Layer 2): The data is packaged into frames for the local network, such as Wi-Fi.

- Physical Layer (Layer 1): Radio signals carry the data over Wi-Fi, or electrical/optical signals carry it over wired links.